# Department of Electrical and Computer Engineering

University of Wisconsin – Madison

ECE 552 Introduction to Computer Architecture

Homework 2 (Due 9/22)

Paste your entire verilog code in a single text/doc file and submit the text/doc file on Canvas. Note that these verilog questions need to follow the Verilog rules specified in the 552 project/verilog rules document that can be found on Canvas.

- Adder/Subtractor

Design a 1-bit full adder, full_adder_1bit, and use it to design a 4-bit Ripple Carry adder+subtractor, addsub_4bit, with an overflow flag to indicate whether overflow has occurred. The circuit has a single bit input to indicate whether it is going to add (0) or sub (1). To implement subtraction, use the “A-B = A+(-B)” and “-B = ~B + 1” properties of binary 2’s complement numbers.

Implement your design in Verilog and provide a testbench for your design to generate random input patterns (hint: use the “$random” system task). The testbench should be self-checking, by comparing the addsub_4bit design’s outputs to a golden result. i.e. you can use ‘+’, ‘-’ in the testbench to check if the outputs of the design under test (DUT) are correct.

The module interface you can use is:

module adder_4bit (
    input [3:0]  a, // input A
    input [3:0]  b, // input B
    input        is_sub, // 1==subtract
    output [3:0] sum, // sum output
    output       ovfl // indicate overflow
);
...
full_adder_1bit fa0(...); // instantiate these bitwise
endmodule // adder_4bit

2. ALU

Design a 4-bit ALU module to implement the following instructions - ADD, SUB, NAND, NOR. Use a 2-bit opcode to identify these instructions.  The opcodes should be used to control the output of the ALU. The add, sub calculations should interface with the module design from Question 1.

You are required to submit the design and a testbench. The design interface to use is:

# module alu_4bit (

# input  [3:0] alu_in1,

# input  [3:0] alu_in2,

# input  [1:0] opcode,

# output [3:0] alu_out,

# output       error // just for overflow

# );

# ...

# endmodule // alu_4bit

# Answers:

# module full_adder_1bit (

# input a,		 // 1 bit input A

# input b,		 // 1 bit input B

# input cin,        	// Carry in

# output sum,             // Sum

# output cout              // Carry out

# );

# assign sum = a ^ b ^ cin;         // XOR produces the sum bit

# assign cout = (a & b) | (a & cin) | (b & cin); 	 // Carry out logic

# endmodule

# module addsub_4bit (

# input  [3:0] a,     		 // 4 bit input A

# input  [3:0] b,     	 	// 4 bit input B

# input  is_sub, 		 // 1==subtract

# output [3:0] sum,    		// 4 bit sum

# output ovfl    		 // indicate overflow

# );

# wire [3:0] comp2_b;

# wire [3:0] cout;    		// carry out of full adders

# assign comp2_b = is_sub ? ~b + 1'b1 : b; 	// two’s complement of b for subtraction

# //instantiate 1 bit full adders

# full_adder_1bit fa0(.a(a[0]), .b(comp2_b[0]), .cin(1’b0),   .sum(sum[0]), .cout(cout[0]));

# full_adder_1bit fa1(.a(a[1]), .b(comp2_b[1]), .cin(cout[0]), .sum(sum[1]), .cout(cout[1]));

# full_adder_1bit fa2(.a(a[2]), .b(comp2_b[2]), .cin(cout[1]), .sum(sum[2]), .cout(cout[2]));

# full_adder_1bit fa3(.a(a[3]), .b(comp2_b[3]), .cin(cout[2]), .sum(sum[3]), .cout(cout[3]));

# // Overflow detection, when the carry into the MSB differs from the carry out of the MSB

# assign ovfl = cout[2] ^ cout[3];

# endmodule





//testbench

# module addsub_4bit_tb();

# reg [3:0] a;

# reg [3:0] b;

# reg is_sub;

# wire [3:0] sum;

# wire ovfl;

# // Instantiate the DUT

# addsub_4bit as4 (.a(a), .b(b), .is_sub(is_sub), .sum(sum), .ovfl(ovfl));

# // Variables for golden result comparison

# reg [4:0] golden_result;

# reg expected_ovfl;

# // Task to check result and overflow

# task check_output;

# begin

# // Calculate the expected result

# golden_result = (is_sub) ? (a - b) : (a + b);

# // Calculate expected overflow

# expected_ovfl = (golden_result[4] != golden_result[3]);

# // Check if the DUT output matches the golden result

# if (sum !== golden_result[3:0] || ovfl !== expected_ovfl) begin

# $display("Error: A=%b, B=%b, is_sub=%b => DUT Sum=%b, Expected Sum=%b", a, b, is_sub, sum, golden_result[3:0]);

# $display("Error: DUT ovfl=%b, Expected ovfl=%b", ovfl, expected_ovfl);

# end else begin

# $display("No Error: A=%b, B=%b, is_sub=%b => Sum=%b, Overflow=%b", a, b, is_sub, sum, ovfl);

# end

# end

# endtask

# initial begin

# // random test cases

# integer i;

# for (i = 0; i < 10; i++) begin

# a = $random % 16;   // Random 4 bit value for A

# b = $random % 16;   // Random 4 bit value for B

# is_sub = $random % 2; // Randomly choose add or subtract

# #5;

# check_output();      // Call the task to check result and overflow

# end

# $finish; // End simulation

# end

# endmodule

# Answers:

# module alu_4bit (

# input  [3:0] alu_in1,

# input  [3:0] alu_in2,

# input  [1:0] opcode,

# output [3:0] alu_out,

# output error // just for overflow

# );

# wire [3:0] add_sub_result;

# wire carry_out;

# wire [3:0] nand_result, nor_result;

# // Instantiate ripple carry adder/subtractor

# addsub_4bit as4 (.a(alu_in1), .b(alu_in2), .is_sub(opcode[0]), .sum(add_sub_result), .cout(carry_out), .ovfl(error));

# // NAND and NOR operations

# assign nand_result = ~(alu_in1 & alu_in2);

# assign nor_result = ~(alu_in1 | alu_in2);

# // ALU output based on opcode

# always @(*) begin

# case (opcode)

# 2'b00: alu_out = add_sub_result; // ADD

# 2'b01: alu_out = add_sub_result; // SUB

# 2'b10: alu_out = nand_result;    // NAND

# 2'b11: alu_out = nor_result;     // NOR

# default: alu_out = 4'b0000;

# endcase

# end

# endmodule

# //testbench

# module tb_alu_4bit();

# reg [3:0] alu_in1;

# reg [3:0] alu_in2;

# reg [1:0] opcode;

# wire [3:0] alu_out;

# wire error;

# // Instantiate the DUT

# alu_4bit alu4 (.alu_in1(alu_in1), .alu_in2(alu_in2), .opcode(opcode), .alu_out(alu_out), .error(error));

# initial begin

# // Test ADD

# alu_in1 = 4'd5; alu_in2 = 4'd3; opcode = 2'b00;

# #10;

# $display("ADD: %d + %d = %d, Error: %b", alu_in1, alu_in2, alu_out, error);

# // Test SUB

# alu_in1 = 4'd7; alu_in2 = 4'd2; opcode = 2'b01;

# #10;

# $display("SUB: %d - %d = %d, Error: %b", alu_in1, alu_in2, alu_out, error);

# // Test NAND

# alu_in1 = 4'd6; alu_in2 = 4'd3; opcode = 2'b10;

# #10;

# $display("NAND: %b NAND %b = %b", alu_in1, alu_in2, alu_out);

# // Test NOR

# alu_in1 = 4'd5; alu_in2 = 4'd2; opcode = 2'b11;

# #10;

# $display("NOR: %b NOR %b = %b", alu_in1, alu_in2, alu_out);

# // Test ADD with overflow

# alu_in1 = 4'd7; alu_in2 = 4'd7; opcode = 2'b00;

# #10;

# $display("ADD (Overflow): %d + %d = %d, Error: %b", alu_in1, alu_in2, alu_out, error);

# // Test SUB with overflow

# alu_in1 = 4'd7; alu_in2 = 4'd8; opcode = 2'b01;

# #10;

# $display("SUB (Overflow): %d - %d = %d, Error: %b", alu_in1, alu_in2, alu_out, error);

# $finish;

# end

# endmodule