# Department of Electrical and Computer Engineering

University of Wisconsin – Madison

ECE 552 Introduction to Computer Architecture

Homework 5 (Due 10/13)

For the question below (Verilog based), paste your entire verilog code in a single text/doc file and submit the text/doc file on Canvas. Note that these Verilog questions need to follow the Verilog rules specified in the 552 project/Verilog rules document that can be found on Canvas.

- Program Counter Control

In this homework, you will focus on the Branch (B) instruction in the WISC ISA (refer to project description) to modify the 16-bit PC. You will need only combinational logic in this module.

Branch instructions conditionally jump to the address obtained by adding the 9-bit immediate (signed) offset to PC+2.

You are provided with 4 inputs: a 1-bit branch flag, 3-bit condition (C), a 9-bit offset (I) and a 3-bit flag (F).

‘c’ specifies the condition as in Table 1 and ‘i’ represents the 9-bit signed offset in two’s-complement representation, but right-shifted by one. The target is computed as: target = PC + 2 + (‘i’ << 1). ‘branch’ is the same signal as seen in CoD 4.24 as a control output and controls the mux selecting between PC+2 and the calculated branch target.

Note: More details on why you do PC+2 and right-shift ‘i’ by 1 are provided in the project description.

Table 1: Encoding for Branch conditions

| ccc | Condition |
| --- | --- |
| 000 | Not Equal (Z = 0) |
| 001 | Equal (Z = 1) |
| 010 | Greater Than (Z = N = 0) |
| 011 | Less Than (N = 1) |
| 100 | Greater Than or Equal (Z = 1 or Z = N = 0) |
| 101 | Less Than or Equal (N = 1 or Z = 1) |
| 110 | Overflow (V = 1) |
| 111 | Unconditional |

The eight possible conditions are Equal (EQ), Not Equal (NEQ), Greater Than (GT), Less Than (LT), Greater Than or Equal (GTE), Less Than or Equal (LTE), Overflow (OVFL) and Unconditional (UNCOND).

As you noticed above, many of these conditions are determined based on the flags N, V and Z. Flag bits are stored in a 3-bit flag (F) register. There are three bits in ‘F’: Zero (Z), Overflow (V) and Sign bit (N). In your project, these flags will be set by arithmetic instructions. But for this homework, you assume that the ‘F’ is an input to your design. The True condition corresponds to the branch being taken. The False condition results in normal PC increment i.e., PC + 2.

The required module is listed below:

module pc_control(input [2:0] c, input [8:0] i, input [2:0] f,
                  input branch, input [15:0] pc_in, output [15:0] pc_out);

Answer:

PROGRAM COUNTER CONTROL DUT

module pc_control(

input [2:0] c,        // condition code (ccc)

input [8:0] i,        // 9-bit signed immediate offset

input [2:0] f,        // flags (F = {N, V, Z})

input branch,         // branch flag

input [15:0] pc_in,   // current PC value

output [15:0] pc_out  // next PC value

);

// Flags

wire N = f[2]; // Sign Bit flag

wire V = f[1]; // Overflow flag

wire Z = f[0]; // Zero flag

// Sign-extend the 9-bit offset to 16 bits and shift left by 1

wire [15:0] offset = { {7{i[8]}}, i } << 1;

// branch condition flag

wire condition = ((c == 3'b000) & (Z == 0)) ? 1'b1 :    // Not Equal (Z = 0)

((c == 3'b001) & (Z == 1)) ? 1'b1 :    // Equal (Z = 1)

((c == 3'b010) & ((Z == 0) & (N == 0))) ? 1'b1 :   // Greater Than (Z = N = 0)

((c == 3'b011) & (N == 1)) ? 1'b1 :    // Less Than (N = 1)

((c == 3'b100) & ((Z == 1) | ((Z == 0) & (N == 0)))) ? 1'b1 :  // Greater Than or Equal (Z = 1 or Z = N = 0)

((c == 3'b101) & ((N == 1) | (Z == 1))) ? 1'b1 :   // Less Than or Equal (N = 1 or Z = 1)

((c == 3'b110) & (V == 1)) ? 1'b1 :    // Overflow (V = 1)

(c == 3'b111) ? 1'b1 : // Unconditional

1'b0;

// Compute the next PC value

wire [15:0] pc_next;

wire [15:0] pc_branch;

adder adder1 (.a(pc_in), .b(16'h2), .sum(pc_next)); // compute PC + 2

adder adder2 (.a(pc_next), .b(offset), .sum(pc_branch)); // compute PC + 2 + offset

assign pc_out = (branch & condition) ? pc_branch : pc_next; //assign pc_branch or pc_next

endmodule

ADDER DUT

module adder(

input [15:0] a,

input [15:0] b,

output [15:0] sum

);

wire [15:0] carry;

wire [15:0] sum_nocarry;

assign sum_nocarry = a ^ b;         // XOR for sum

assign carry = (a & b) << 1;        // AND for carry and shift left

assign sum = sum_nocarry ^ carry;  // Final sum with carry

endmodule

TESTBENCH

module pc_control_tb();

reg [2:0] c;           // condition code (ccc)

reg [8:0] i;           // 9-bit signed immediate offset

reg [2:0] f;           // flags (F = {N, V, Z})

reg branch;            // branch flag

reg [15:0] pc_in;      // current PC value

wire [15:0] pc_out;    // next PC value

// Instantiate UUT

pc_control uut (.c(c), .i(i), .f(f), .branch(branch), .pc_in(pc_in), .pc_out(pc_out));

initial begin

// Initialize Inputs

i = 9'b000000001; // Immediate offset = 1

branch = 1; // Enable branch

pc_in = 16'h0000; // Current PC = 0

// Test Case 1

c = 3'b000; // Not Equal condition

f = 3'b000; // Flags: {N, V, Z} = {0, 0, 0}

#10;

$display("Test Case 1: c=%b, i=%b, f=%b, branch=%b, pc_in=%h => pc_out=%h", c, i, f, branch, pc_in, pc_out);

// Test Case 2

c = 3'b001; // Equal condition

f = 3'b001; // Flags: {N, V, Z} = {0, 0, 1}

#10;

$display("Test Case 2: c=%b, i=%b, f=%b, branch=%b, pc_in=%h => pc_out=%h", c, i, f, branch, pc_in, pc_out);

// Test Case 3

c = 3'b010; // Greater Than condition

f = 3'b000; // Flags: {N, V, Z} = {0, 0, 0}

#10;

$display("Test Case 3: c=%b, i=%b, f=%b, branch=%b, pc_in=%h => pc_out=%h", c, i, f, branch, pc_in, pc_out);

// Test Case 4

c = 3'b011; // Less Than condition

f = 3'b100; // Flags: {N, V, Z} = {1, 0, 0}

#10;

$display("Test Case 4: c=%b, i=%b, f=%b, branch=%b, pc_in=%h => pc_out=%h", c, i, f, branch, pc_in, pc_out);

// Test Case 5

c = 3'b100; // Greater Than or Equal condition

f = 3'b010; // Flags: {N, V, Z} = {0, 1, 0}

#10;

$display("Test Case 5: c=%b, i=%b, f=%b, branch=%b, pc_in=%h => pc_out=%h", c, i, f, branch, pc_in, pc_out);

// Test Case 6

c = 3'b101; // Less Than or Equal condition

f = 3'b100; // Flags: {N, V, Z} = {1, 0, 0}

#10;

$display("Test Case 6: c=%b, i=%b, f=%b, branch=%b, pc_in=%h => pc_out=%h", c, i, f, branch, pc_in, pc_out);

// Test Case 7

c = 3'b110; // Less Than or Equal condition

f = 3'b010; // Flags: {N, V, Z} = {0, 1, 0}

#10;

$display("Test Case 7: c=%b, i=%b, f=%b, branch=%b, pc_in=%h => pc_out=%h", c, i, f, branch, pc_in, pc_out);

// Test Case 8

c = 3'b111; // Unconditional condition

#10;

$display("Test Case 8: c=%b, i=%b, f=%b, branch=%b, pc_in=%h => pc_out=%h", c, i, f, branch, pc_in, pc_out);

//Test Case 9

branch = 0; // Disable branch

#10;

$display("Test Case 8: c=%b, i=%b, f=%b, branch=%b, pc_in=%h => pc_out=%h", c, i, f, branch, pc_in, pc_out);

// End the simulation

$finish;

end

endmodule

WAVEFORM